Wireless Router Physical Setup – Step-by-Step

1. Connect Power

- Plug the DC power connector into the wireless router and turn it on.

2. Reset the Router

- Using a pin, press and hold the reset button on the router for at least 3 seconds to reset it to factory settings.

3. Connect the Internet Source (WAN)

- Take the main Ethernet cable (coming from your ISP switch or modem) and plug it into the WAN port of the wireless router (usually blue-colored).

4. Connect to Your PC (LAN)

- Connect a second Ethernet cable from one of the router’s LAN ports to your computer’s Ethernet port.

Important: Both Ethernet cables must be connected at the same time. After connecting, your computer's internet may temporarily stop.

5. Configure Network Settings on PC

- Press Windows + R → type ncpa.cpl → press Enter.

- Right-click on your Ethernet adapter (e.g., Realtek PCIe) → click Properties.

- Find Internet Protocol Version 4 (TCP/IPv4):

  - Uncheck it, then check it again.

  - Click Properties.

  - Select:

    - Obtain an IP address automatically

    - Obtain DNS server address automatically

  - Click OK to save.

✅ Your system’s internet will be restored after this step.

6. Check IP Address Details

- Right-click your Ethernet adapter → click Status → click Details.

- You will see details like:

  - IPv4 Address

  - Default Gateway

  - Subnet Mask

- Example: 192.168.0.1 (This will be your router's login IP).

7. Login to Router

- Open your web browser and type the Default Gateway IP (e.g., 192.168.0.1) in the address bar.

- You’ll be directed to the router’s login page (e.g., TP-Link, CP-Plus, etc.).

- Enter the default username and password (usually written on the router’s label).

8. Set Up Wireless Router

- Follow the on-screen steps to:

  - Set SSID (Wi-Fi name)

  - Set Wi-Fi password

  - Configure any additional settings (security mode, etc.)

- Save and reboot the router if required.